LIMA-NODE

nRF52840 MDK USB Dongle — NCS/Zephyr Flash Guide

NCS v3.2.0-rc1  |  Zephyr 4.3.99  |  Written Feb 2026

# Overview

This document captures the exact steps required to build and flash Zephyr/NCS firmware onto the Makerdiary nRF52840 MDK USB Dongle. The dongle does NOT have an onboard J-Link debugger, so the standard west flash workflow requires a one-time board.cmake fix to use the UF2 runner instead.

Hardware: Makerdiary nRF52840 MDK USB Dongle

Board target: nrf52840_mdk_usb_dongle/nrf52840

SDK: Nordic Connect SDK (NCS) v3.2.0-rc1 on top of Zephyr 4.3.99

# 1. Environment Setup

## 1.1 Workspace Layout

The NCS workspace uses west as its meta-tool. The expected layout after west init and west update:

~/a/m/                    ← workspace root

lima-node/              ← your manifest repo

zephyr/                 ← Zephyr RTOS

nrf/                    ← Nordic Connect SDK

bootloader/             ← MCUboot

.venv/                  ← Python virtual environment

## 1.2 Python Environment

Always activate the virtual environment before building:

cd ~/a/m

source .venv/bin/activate

⚠️  If west commands fail with import errors, you forgot to activate the venv.

# 2. One-Time Board Fix (Critical)

Out of the box, the MDK USB Dongle board definition points to the nrfjprog runner which requires a J-Link. The dongle uses UF2 bootloader instead. You must patch the board CMake file once:

## 2.1 Edit board.cmake

nano ~/a/m/zephyr/boards/makerdiary/nrf52840_mdk_usb_dongle/board.cmake

Change the runner line to:

include(${ZEPHYR_BASE}/boards/common/uf2.board.cmake)

✅  Without this fix, west flash will fail with 'runner not configured' or attempt to use nrfjprog which will not find the dongle.

# 3. Building Firmware

## 3.1 Build Command

Always build from the workspace root, not from inside the sample directory. The -DCONFIG_BUILD_OUTPUT_UF2=y flag is required — without it the UF2 runner cannot find the output file.

cd ~/a/m

west build -b nrf52840_mdk_usb_dongle/nrf52840 zephyr/samples/basic/blinky \

-- -DCONFIG_BUILD_OUTPUT_UF2=y

## 3.2 Pristine (Clean) Build

Use -p always to wipe the build directory and start fresh. Safe to use anytime:

west build -p always -b nrf52840_mdk_usb_dongle/nrf52840 \

zephyr/samples/basic/blinky -- -DCONFIG_BUILD_OUTPUT_UF2=y

⚠️  Never manually rm -rf the build dir unless you want to regenerate everything. Use -p always instead.

## 3.3 Sysbuild Note

Some samples (like blinky_pwm) use sysbuild which generates a merged.hex and can complicate the flash workflow. If a sample uses sysbuild and west flash fails, add --no-sysbuild to the build command:

west build --no-sysbuild -b nrf52840_mdk_usb_dongle/nrf52840 \

zephyr/samples/basic/blinky_pwm -- -DCONFIG_BUILD_OUTPUT_UF2=y

# 4. Flashing

## 4.1 Enter Bootloader Mode

The dongle must be in UF2 bootloader mode before flashing:

- Hold the RESET button while plugging the dongle into USB

- OR double-tap RESET if already plugged in

- The RGB LED will pulse RED indicating bootloader mode

- The dongle mounts as a USB drive at /media/<user>/UF2BOOT

## 4.2 Flash

Once the dongle is in bootloader mode and mounted:

west flash

West will automatically copy the .uf2 file to /media/<user>/UF2BOOT. The dongle will reboot and run the new firmware.

✅  The LED stops pulsing red and returns to normal behavior once flashing is complete.

# 5. Validating the Toolchain

Use this progression to validate each layer of the stack before working on LIMA-specific firmware:

| Sample | Path | Validates |
| --- | --- | --- |
| blinky | zephyr/samples/basic/blinky | Toolchain, west, J-Link/UF2, GPIO |
| blinky_pwm | zephyr/samples/basic/blinky_pwm | PWM peripheral (use --no-sysbuild) |
| bluetooth/peripheral_hr | nrf/samples/bluetooth/peripheral_hr | BLE stack, SoftDevice, advertising |
| bluetooth/peripheral_lbs | nrf/samples/bluetooth/peripheral_lbs | BLE bidirectional comms (closest to LIMA) |

# 6. Troubleshooting

### west flash fails: 'runner not configured'

You haven't patched board.cmake yet. See Section 2.1.

### FATAL ERROR: uf2 file location is unknown

You forgot -DCONFIG_BUILD_OUTPUT_UF2=y in the build command. Rebuild with that flag.

### ninja: error: loading 'build.ninja': No such file or directory

The build directory is stale or from a different sample. Run a pristine build with -p always.

### west flash copies the file but dongle doesn't reboot

The dongle wasn't in bootloader mode when you ran west flash. Hold reset and plug in again, verify the LED pulses red, then reflash.

### Build fails at [40/202] or similar mid-build

Likely a sysbuild conflict. Add --no-sysbuild to the build command.

LIMA-NODE — Local Integrity Multi-modal Architecture